Day 1: Build the robot world in MATLAB (no Python yet)

- 1.1 Workspace. Create a project folder and inspect the robot's links and joints.
Done when: you can see the joint and end-effector names.

- 1.2 Read the gripper position. Get the end-effector's x, y, z at the home pose.
Done when: MATLAB prints three numbers in metres.

- 1.3 Safety limits. Three layers:

  - Joint limits, from the model, plus a small margin.

  - A workspace box: the gripper tip may only be inside an allowed x/y/z range. This copies the idea of the protection zones you made on the real arm.

  - A speed limit, kept slow for testing.

Done when: a target outside the box is rejected with a clear reason.

- 1.4 Up/down motion. Move the tip up 10 cm and back down using inverse kinematics (working out joint angles from a target position) and a smooth trajectory, animated in the figure.
Done when: you watch the arm go up and down.

- 1.5 One reusable function. Wrap it all in sim_move_vertical(height), which returns "done" or "blocked" plus a reason.
Done when: safe and unsafe heights both give the right answer.

Day 2: Connect the agent

- 2.1 MATLAB listens. MATLAB opens a local server and waits for JSON commands such as {"command": "move_updown", "height_m": 0.10}.

- 2.2 Test with a plain Python script. No agent yet, just proving the link. Send a safe command, then an unsafe one.
Done when: the arm moves, and the unsafe one is rejected.

- 2.3 Plug into your agent. We add a "MATLAB backend" to the simulated hardware layer you already built. I'll need to see that file first so I don't guess its structure.

- 2.4 Agent-driven demo. The agent calls its tool, the arm moves in MATLAB, and the task logger records the result.

- 2.5 Failure tests. Unsafe target, MATLAB not running (timeout), and safe_stop. This gives you a small results table for your report.